Project Name: spinz

Target Category: City / Urban Single Speed

Frame Material: aluminum

### 1. Frame & Fork Specifications

- Frame: aluminum , Urban Geometry.

- Fork: steel- Straight blade.

- Welding: Standard TIG Welding.

- Anti-Rust Treatment: ED Coating (Electrophoretic Deposition) – Required for durability.

- Dropouts: Horizontal track dropouts (for chain tensioning).

- Cable Routing:  internal

### 2. Drivetrain (Single Speed Setup)

- Chainwheel: 46T Steel/ Crankset (170mm arms).

- Rear Cog: 16T Freewheel (Single Speed).

- free Hub: free wheel

- Chain: KMC Z410 (Heavy duty for single speed).

- Bottom Bracket: Sealed Cartridge BB, BSA Threaded 68mm.

### 3. Components

- Brakes: Dual-Pivot Side-Pull Caliper Brakes (Front & Rear).

- Brake Levers: Alloy 2-finger levers.

- Handlebar: [Flat bar], Alloy, 25.4mm or 31.8mm.

- Stem: Alloy, 60mm/80mm extension.

- Seatpost: Alloy,  35 centimeter with integrated clamp.

- Saddle: Urban Comfort

- Pedals: hard plastic platform pedals.

### 4. Wheels & Tires

- Rims: 700c Double-Wall Aluminum Alloy, [Specify depth: 30mm].

- Hubs: High-flange steel, 32H, Nutted axle.

- Tires: kenda 5mm iner layer for extra protection

Tire Requirements:

high-quality, puncture-resistant tires for urban use

Detailed Specifications:

- Size: 700 x 32c (Preferred) or 700 x 28c.

- Sidewall: Must include a Reflective Strip(Reflective Sidewall) for night safety.

- Inner Tubes: Please equip the bikes with Self-Sealing Inner Tubes (pre-filled with sealant/liquid) to prevent flats.

- Valve Type: Presta Valve (French valve) suitable for 700c rims.

- Tubes: Butyl rubber, Presta or Schrader valve.

### 5. Finish & Branding

- Paint: Single tone powder coating.

- Color:

- Decals: Under-clear coat (Water-transfer).

- Head Badge:

### 6. Compliance & Safety (Mandatory for Israel)

- Safety Standard: Must comply with ISO 4210-2 (City/Trekking).

- Test Report: Manufacturer must provide a valid test report from SGS, TUV, or Intertek.

- Accessories Included: * Front and Rear Reflectors.

  - Wheel Reflectors (White/Amber).

  - Pedal Reflectors.